A History of Oaken Shipbuilding: From Viking Longboats Through Britain’s Age of Sail.

Tie A Yellow Ribbon Round the Ole Oak Tree was a popular hit by Tony Orlando and Dawn in 1973.  It told the story of someone who was imprisoned and desired a sign upon his return from his lover to know if he was still wanted in her life.  It has become a symbol of the return of someone who is missing or lost.  During the Seven Years War 66 British naval vessels sailed off to sea and just disappeared.  The ships were not casualties in battle or catastrophic storm.  In high waves, the ship can be lifted up and pounded, shivering or splintering the structural framework of supporting timbers.  Most likely, they just sailed off their wooden bottoms in rough seas because the rotten oak timbers between wind and water gave way----the ships perished with their crews sinking like cobblestone--- or perhaps surprisingly like a log of green, oak wood.  Oak is less than one third as dense as fresh water and should float but it is not as light as pine and fir and under everyday conditions it struggles to stay afloat especially when newly cut.  A twenty-foot oak log two feet in diameter weighed over 3500 pounds and the butt of the log would be ‘tushed’ to the harnesses of large oxen and draft horses.  Twenty miles was the maximum distance that such timber to be feasibly dragged to a beach where they could be floated to dockyards and sawmills after being rafted together with lighter, pine logs for greater buoyancy.  Otherwise, the oaken logs at greater expense could be barged down rivers to coastal waters.  This is the origin of someone who stubbornly sticks to a difficult task, as they are ‘in it for the long haul.’  Such limitations did not prohibit it from being used in shipbuilding and in the Age of Sail about 90% of all ships were constructed of oakwood.  Iron and steel don’t float either unless shaped in a way that displaces enough water to be buoyed with a force that exceeds its weight.   Oak was prized in shipbuilding because it was not too heavy, fairly pliable, strong and being nonporous, resistant to the penetration of water.  Large oak vessels constructed by the Royal Navy contained below deck cavernous holds which were ventilated adequately when the ships were sailing at sea.  But when they were ‘laid up in ordinary’ during peacetime the structural timbers just above the waterline were sometimes wet and sometimes dry.  They were in the wrong spot subject to the decay from fungi; it was not uncommon that toadstools as big a sailor’s fist would grow.  A crew was sent to shovel the mushrooms out holds of ships prior to inspection by the British Admiralty.  Fungi need a balance of air and moisture to thrive and just at the waterline conditions are optimal for their growth.  More fungi, more rot: enemy gun crews often targeted right above an opponent ship’s waterline because it was known that it was a weak seam that their cannon balls could puncture.

Alone among the trees of the temperate forest, an oak, shoots out immensely long, sturdy branches that typically are 90 degrees from its vertical trunk.  When the trees grew in dense, mixed, compacted woodlands the trunks would rise straight with few branches until near the treetop crown.  Under such conditions, the branch crotches grow wide but strong and function well in such forking shapes for compass timbers.  The search for sunlight governed the arboreal architecture.  When oak trees grew in the open, pasture, glade or meadow---- the tree had wide-reaching horizontal branches from bottom to top.  Where the tree grew on the landscape was a crucial sign to its health and its value for usable timber.  Did it grow in thin soils with projecting rocks? If so, it would be less sound for it grew under more stress.  A windblown tree usually had decay in its base or roots that usually due to a natural impediment such as ledge or a high-water table penetrated the soil only shallowly.  A tree feller had to have a sharp eye for detail and it was backbreaking work where even with experience the results were often heartbreaking.  And it was dangerous!  A trained feller could make a tree when cut fall where he wanted but if the tree was flawed within by a hidden defect it could split, twist like a swiveling barbering chair and fall unpredictably.  At times men were injured or killed and timber ruined for everything but fire kindling.  And the wise tree-feller knew that at the final moment---when the chopped tree should fall—there was an illusion which was deadly.  The tree lingered, hesitated, not appearing to fall at all. “Why isn’t it falling?”  If its inertia was perceptibly too long, a crafty, woodsmen would quickly run, bolting to one side or another, for he knew that the only spot where a tree does not appear to fall is in the direct line to where it is falling.  Other disappointments were less dramatic but nonetheless cruel.  The sawn crosscuts might expose a blackened heart---rotting away that would make the log unsuitable for lumber.  When hewn, a small cup-sized shake might widen and spread longitudinally revealing unsound cracks along its whole length.  We describe unreliable, weak people as ‘shaky’ and this is where the term comes from.

Nothing went to waste.  The feller looked at the shipwright’s molds and templates, then sought to find similar shapes in the trees.  From the felling ax, he changed to a broad ax to hewn out a timber for the ship’s frame.  Figuring out how broad a timber he wanted he would cut notches at the same depth along the length, then he would cut crossway chopping away the notches until he had a smoother face, then he would flip the timber with a cant hook and perform the same task on the opposite side.  It would become a rib for the ship’s framing.  The skeleton of each ship called for hundreds of structural support pieces which were scarfed, mortise-and-tenoned, lapped and at times bolted together.  Only when the frame was raised into place would it be covered with boarding to create its shell.  This process was opposite of how the Vikings had built their northern ships.  A Viking vessel was 90% planking and only 10% framing; a supporting skeleton that came last in the construction of the Viking longship.  The frame was fitted into the completed shell and fastened by lashing or with wooden spikes called ‘trenails.’  And wherever the Norsemen found oak, it was the lumber used.  The planking for the side strakes were spilt from whole logs.  A tree trunk three feet in width could yield up to 20 usable boards.  No wood was sawn in Europe until the 12th century and a cloven board possessed twice the strength of a sawn board.  When the wood grain was true and ran straight a skilled woodcutter could possibly make planks as long as 20 feet, but usually the boards were less than a dozen feet.  Oak can be split into boards lengthwise but also from the ends of the log back toward its center.  The up-and-down, vertical fibers of the oak tree are crossed by rays of living cells that run horizontally inward from the outer bark to the center pith, stabilizing the internal framework.  By following the grain of the rays one can clove off wedge shape pieces.  The Norsemen artisans who made longboats were illiterate but they realized that if you overlapped the boards----thin part of one wedge to the thick edge of another---and nail them together with forged, iron rivets—the shell of the longboat fit together to make a vessel that was inherently light, streamlined, shallow drafted and waterproof.  For its weight, no better ship than what the Norsemen designed was ever built.  Under row or sail, it could turn on a coin and it drew only three feet of water allowing it to explore all but the shallowest bays, estuaries and rivers.  It could make 20 knots under its square sail and about five knots when rowed.  Strong and resilient, but light enough to be hauled up onto sandy beaches by its sailors.  Building always began with the stem and the keel. The Norse boatbuilder was called a stem-smith because the stern at the rear and prow—the most forward part of a boat’s hull—were the cornerstones of the Scandinavian design.  Using a knotted cord, a stem-smith measured one-third the length of the keel.  Then at that point he traced a circle upward to obtain its radius for the shape of the circular stem which was carved from one large single piece of oak.  For better boats, it was a three-dimensional functional piece of artistic beauty.  It had a sharp, rising forward edge with two wings located behind incised with the exact angle for the rabbeting of the planking.  A leaky boat wouldn’t float which was deadly in deep, icy waters.  Even a well-crafted vessel sinks down in the water as the boat gathers way.  The forward motion is opposed by the bow and stern.  The trough of the bow wave when it reaches the length of the boat causes it to squat very low in the water.  If more power is applied, the forward thrust causes the vessel to rise up and efficiently plane.  Also, the boatbuilder had to avoid getting the center of gravity too high because the boat would wallow being slow to right itself.  The helmsman when steering would experience endless bouts of troubling jerking back and forth—the boat was said to be ‘cranky.’  An irascible, unreliable, moody person is said to be cranky and that unflattering connotation comes from this nautical term a boat that was hard to handle when underway.

More than half of all the wooden ships ever constructed were built in the first 75 years of the 19th century—during the latter years of the Age of Sail.  Unlike Norsemen longboats, these ships were too large, too complex and too costly to chance to building by the carpenters’ rule of thumbs.  The shipwright would carve, construct and assemble a small-scale model of the design that was proposed from prow to stern.  Clients, shipowners would squabble over changes but once the model was accepted it served as the 3-dimensional blueprint for construction.  The design model would be given to the lofts-man, who would meticulously sketch out full size templates for each ship piece on thin sheets of wood.  These templates would be given to the boat-builder who looked for an appropriate piece of timber onsite to match what was needed.  Timber-getters surveilling the forests would get templates as they searched trees for the required sizes and shapes.  Trees had to be found that had grown straight-grained to the necessary shapes and sizing especially for oak lumber that was never grossly bent.  Hundreds if not thousands of pieces had to be cut precisely so that the skeleton would provide a sturdy frame where each jig-sawed piece fit snugly interlocked with its neighbor to prevent leaking.   Deforestation of the English woodlands and shortages of timber since the Elizabethan times caused a major acquisition concern for the British Admiralty.  Long before any shipyard construction took place were years of forethought and planning that invested heavily in the planting of the necessary acreage of oak and elm.  Appropriate tree species were planted, cultivated, pruned, selected, cut and the wood stacked to season properly.  Arborists deformed trees so that they grew in a curved manner.  An oak tree might mature in sixty years and then the cut green wood required seasoning to dry that required two to three years to become construction wood ideal for shipbuilding.  It was a deliberate, long-term and rather rigid procurement process and despite great care and effort by the mid-1700s the English countryside had exhausted all supplies where transport from the woodlands to the dockyards was economically feasible.  To keep a vessel in service, a ship of the late 18th century was in a state of constant reconstruction involving intense efforts at repair and/or replacement of rotted wooden parts so by the end of a ship’s life scarcely little remained of the original woodworking.  Many types of trees could not supply lumber suitable for marine vessels and on any ship the varied woods used deteriorated at different rates.   At urgent times, especially when at war, green timber was used in desperation but such haste made vessels unseaworthy.  Unseasoned wood prompted boats to begin to rot even before they were launched.  The materials to construct and outfit ships during the Age of Sail [1571 to 1862] included prolific quantities of not only lumber and masts, but also iron, hemp for rope, gunpowder, pitch and tar and by the late 1700s copper ore was mined and smelted for the British had begun plating the bottoms of their ships to protect against fouling and destructive seaworms while at sea.  Acute shortages of timber for shipbuilding occurred between 1775 and 1815 and since it was a key part of Britain’s sea-power, and its security, is agents searched the world for new, suitable timber supplies.  The average oak harvested for timber made up 50 cubic feet or one ‘load’ of lumber weighing one ton.  Taking four years to complete, an average 74-gun British Navy ‘ship of the line’ took 3, 000 loads of lumber to construct which was equal to 2,000 oak trees that required about what just less than sixty acres of woodland could produce at tree maturity.  The British Navy classified their ship of the line as being a three masted, squared-rigged vessel that carried at least sixty cannons on board; this standard was the minimum firepower that was required to form a continuous line of vessels that could attack an enemy fleet broadside while providing cover for one another as they progressed down the line of the opponent’s ships.  Such ships often were deployed in this ‘line ahead’ attack and defense strategy.  Ships with fewer than sixty cannons were cruisers or frigates; first rate ships carried ninety or more big guns, second rate carried 80 to ninety, third rate ships had 64 to 74 guns.  Fourth rate ships usually frigates carried fifty guns.  Fifth and sixth rate ships were classified as cruisers that carried less than forty guns but not less than twenty-four.

The British Navy Board was prejudice about the perceived inferior quality of foreign sourced wood and was fearful that reliance on sources outside of Britain was a strategic liability—but scarcity forced the recalcitrant British to reconsider.  First foreign sources supplied wood was only employed for repair of ships but soon it was being utilized for new construction as well.  By the late 18th century, all the lands whose rivers drained into the Baltic Sea as well as the seas adjacent to Scandinavia and Arctic Russia were supplying needed timber including the tallest trees for great masts for the British Navy.  Norway, favored by its proximity to England, was a big supplier of timber during the 18th century and by the early 19th century its own vast forests became overexploited.  The eastern Baltic furnished oak, and ship masts made from tall spruces and firs.  Holland and Germany supplied oak and the American colonies supplied both oak and pine although due to the high shipping costs the supplies were limited especially compared to the Norwegian supply.  Finland, Russia and Sweden not only were purveyors of wood but pitch and tar that was needed for caulking and for the ship’s cordage.  The ropes of the standing rigging attached to the spars and masts were well-tarred with what was called ‘Stockholm tar.’  Britain was lacking in enough conifer trees that supplied pitch and tar and the Swedes had such a monopoly that the pine forests in the colony of North Carolina were tapped as an alternative supply.  Today we still refer to this American source if only in a rather obtuse manner, recall the tar-heels from the University of North Carolina.  Despite its promise due to what appeared to been an inexhaustible supply, the quality was poorer and despite incentives in times of foreign embargo, the American supply never did displace the supply from Sweden because of the higher transportation costs.  British ships to Baltic ports easily could make several times more trips there compared to one or perhaps at best two annually to the Americas.  In 1775, as the conflict heated up in the American Colonies, exports to Britain from the American South stopped and the British explored the feasibility of northern colonial sources of the pitch and tar to augment the Scandinavian sources.  Eventually, Britain resorted to searching the entire world for timber during the Napoleonic Age due to restrictions imposed by the Continental System.  In 1807, after timber imports were restricted from the Baltic, Canadian also exported more masts to Britain annually, reaching a maximum of over 23,000 shipped by sea in 1811.

Wooden hulls were covered with ‘white stuff’ during most of the 17th century; it was a waterproofing composite to protect the wood consisting of train oil made from bowhead whales, or oil rendered from seals or fish, sulfur and rosin derived from pine trees.  By the 18th century most ship bottoms were coated with a cheaper waterproofing alternative called ‘black stuff.’  It was composed of two parts pitch to one of tar.  The tar was made by the destructive distillation of pine wood to make charcoal.  Both were ultimately derived from pine trees---- the more liquid pine tar or raw pine sap were boiled down to make the more viscous, harder pitch.  During the War for Independence, Canada supplemented pitch and tar replacing the American suppliers so that Britain’s reliance on Swedish sources was never as high as in the 1700s.  In the 18th century, the Royal Navy began using copper sheathing to protect their ships from the wood-burrowing teredo or shipworm, which is not a worm at all but rather a clam-like mollusk that is most prevalent in warmer seas.  In 1761, experiments with copper sheathing were tried on the bottom of the HMS Alarm as it was being outfitted for a two-year cruise to the West Indies.  It had previously returned to England, having suffered extensive damage due to the teredo worm.  The larvae of the misnamed ‘worm’ drill into submerged wood, then burrow along the grain as they grow, turning the wood shavings with the help of bacteria into both food and a protective shell.  They can also filter plankton, breathe, excrete and breed with a pair of occasionally protruding siphons through the first, small borehole all while hollowing out worm-like holes within the wood.  The Essex, the 1820 whaling ship that inspired Herman Melville’s Moby Dick, was rammed, stoved and sunk by an enraged bull sperm whale was thought to have been compromised by shipworm damage.  The copper not only prevented the infestation of a wooden hull at sea but kept it clean from weed and algae growth improving the vessel’s performance under sail.  There, however, existed a little-understood galvanic reaction between the iron bolts used in a hull’s original construction with the copper itself which severely compromised the iron components.  It wasn’t until 1783 when the iron bolts used as fasteners were replaced with an alloy of copper and zinc that the corrosion problem was solved, leading to widespread use of copper sheathing on all ships of the Royal Navy.  To this day copper-bottom, copper-lined or copper-toped items are marketed as ensuring superior reliability and craftsmanship.

Songs were written about the national tragedy in 1782 when the 100-gun, 180 feet length, 2000 tons, first-rate ship of the line, HMS Royal George sunk when the great ship lost its bottom while being heaved over for minor repairs; Admiral Richard Kempenfelt and over 800 people were drowned when the ship capsized in port.  ‘The bottom fell out’ was a naval term that originated with the tragedy and it survives today to describe rapid, unanticipated disaster not in ships but in the equities market.  The magnificent ship that took ten years to construct saw significant action in the Seven Years War.  On August 29th, in early morning she was receiving stores including kegs of rum from tenders, when laying at anchor at Spithead Anchorage near Portsmouth.  In calm water in sight of the shoreline, shipwrights, carpenters and dockyard plumbers began repairing a cistern pipe that drew in seawater for washing the wooden decks.  The inlet of the submerged pipe was 3 feet below the waterline so the carpenter requested that the ship be heeled over.  The lower deck heavy guns on port were extended out while on the starboard side they were rolled in to shift the weight of the ship.  The leaning was minimal so the upper deck guns repeated the same procedure and on the middle deck three of the guns were rolled to portside; the ship listed sufficiently for the work to progress.  After breakfast, the carpenter reported to the 19-year-old Lieutenant Philip Charles Durham of the ship’s watch that the ship was precariously settling deeper into the water; at first the officer ignored him but the carpenter persisted and Captain Waghorn heeding the warning gave orders to shift the guns in order to the level the ship.  A gust of wind caused the ship to list further causing many of the starboard guns to roll to port and water began to pour in.   Drummers were immediately commanded to beat to quarters as a signal for corrective action but minutes later without any apparent righting, or warning, the bulwark supports of the sidewalls gave way, and the ship suddenly sank.  The huge ship rolled on her side until her masts lay horizontal on the still water; the great, mainmast bearing down on the sloop Lark moored alongside.  The sloop was dragged down to the bottom too.  Admiral Kempenfelt who was being shaved and barbered in his quarters below perished when his room door jammed as the ship rolled.  Captain Waghorn and the lieutenant who caused the delay like the 255 others pulled from the water survived because they had stood on open decks but most of those trapped below perished.  Only seventy fortunate souls were able to escape from below deck through open ports.  At the hearing conducted later to investigate the loss of life and the ship, evidence was presented of several beams from salvage that were rotted and caused the ship’s frame to collapse under the stress of the heel.  There were in addition to the crewmen some 300 women and 60 children on board.  Many were family members treasuring their last moments together before their men took leave but some of the women were later described euphemistically as ‘ladies who though seeking neither husbands nor fathers, yet visited our newly arrived ships of war.’  The scene below deck was an amalgam of the poignant emotion of goodbyes, send-off and debauchery that reflected the reality of the hard lives of the times.  Several days later bloated bodies started to bop up and the watermen who ferried families, purveyors and traders to the ships scavenged the bodies for valuables before rafting them together like floating timber.  The dead bodies were carted to an excavation site dug for them in nearby Kingston churchyard for their final interment.  A yellow ribbon perhaps tied around the perimeter oaks at the gravesite would had been appropriate.

By R. Anthony Saritelli			December 21, 2020—the Winter Solstice.

References:

Bryant Logan, W. Oak. The Framework of Civilization.  W.W. Norton & Co. New York. 2005.

Marston, D. Essential Histories: The Seven Years’ War. Osprey Publishing. Oxford, Great Britain. 2001.

Corbett, J.S. LL.M. England in the Seven Years War. Volume II. Longmans, Green & Co. London. 1907.

Oster, R.M. Great Britain in the Age of Sail: Scarce Resources, Ruthless Actions & Consequences.  Air Command & Staff College. Alabama, USA April, 2015.

Teredo Worm-Wikipedia

Age of Sail-Wikipedia

The Loss of the HMS Royal George: The National Museum. Royal Navy Library. 2000.

Dawlish Chronicles-Short Stories. The Loss of HMS Royal George 1782.